Long-term VHE gamma-ray monitoring

of bright blazars

with a dedicated Cherenkov telescope

Letter of Intent

Draft, February 2007

Karl Mannheim, Thomas Bretz, Wolfgang Dröge,

Daniela Dorner, Markus Meyer

Universität Würzburg

Wolfgang Rhode, Michael Backes, Julia Becker, Jan Lünemann

Universität Dortmund

Adrian Biland, Eckart Lorenz

ETH Zürich

Riccardo Paoletti

Università di Siena and INFN sez. di Pisa, Italy

Abstract: We propose to set up a Cherenkov telescope with low-cost but high performance design for robotic operation. The goal is to achieve long-term monitoring of bright blazars which will unravel the origin and nature of their variability. The telescope design is based on a technological upgrade of one of the former telescopes of the HEGRA collaboration on the Canarian Island La Palma (Spain).

Introduction: Since the termination of the HEGRA observations, the succeeding experiments MAGIC and HESS have impressively extended the physical scope of gamma ray observations by detecting tens of formerly unknown gamma ray sources and analyzing their energy spectra and temporal behavior. This became possible by lowering the energy threshold from 700 GeV to less than 100 GeV and increasing at the same time the sensitivity by a factor of five.

To fully exploit the discovery potential of the improved sensitivity, the discovery of new, faint objects has become the major task for the new telescopes. A diversity of astrophysical source types such as pulsar wind nebulae, supernova remnants, microquasars, pulsars, radio galaxies, clusters of galaxies, gamma ray bursts, and blazers can be studied with these telescopes and limits their availability for monitoring purposes of well-known bright sources.

A consequence of the large demand for observing many objects is that observation time is too precious to carry out long term monitoring of rarely flaring sources.

There are strong reasons to make an effort for the continuous monitoring of the few exceptionally bright blazars. This can be achieved by operating a dedicated monitoring telescope of the HEGRA-type, referred to in the following as DWARF (Dedicated multiWavelength Agn Research Facility). The reasons are outlined in detail below.

The science case: The variability of blazars, seen across the entire electromagnetic spectrum, arises from the dynamics of relativistic jets and the particle acceleration going on in them. The jets are launched from the vicinity of accreting supermassive black holes, and theoretical models predict variability arising from the interplay between jet expansion, particle injection, acceleration and cooling. An understanding of this variability will deepen our knowledge about

- the composition and generation of the jets, intimately connected to the physics of the ergosphere of rapidly spinning black holes embedded into the hot plasma from the accretion flow.

- the plasma physics responsible for highly efficient particle acceleration, bearing similarities to plasma physics of the interaction between extremely intense laser beams and matter.

- the orbital modulation of jets due to binary black holes expected from galaxy merger models.

Long-term monitor observations of bright blazars are the key to obtain a solid data base for variability investigations. Assuming conservatively the performance of a single HEGRA-type telescope, long-term monitoring of at least the following blazars is possible: Mrk421, Mrk501, 1ES 2344+514, 1ES 1959+650, H 1426+428, PKS 2155-304. We emphasize that DWARF will run as a facility dedicated to these targets only, providing a maximum observation time for the program. At least one of the proposed targets will be visible any time of the year (see plot/appendix). For calibration purposes, some time will be scheduled for observations of the Crab nebula, which is the brightest known VHE emitter with constant flux.

High-peaked BL Lacertae objects are the prime source population for studies with Cherenkov telescopes.  It is obvious that monitoring observations of strong blazars are orthogonal to the mission of the larger Cherenkov telescopes with their discovery potential for new sources (luminosity function, redshift distribution).

In detail the following investigations are planned:

- As direct result of the measurements, the duty cycle, the baseline emission, and the power spectrum of flux variations will be determined and compared with variability properties in other wavelength ranges.

- The lightcurves will be interpreted using models for the nonthermal emission from relativistically expanding plasma jets. In particular models currently developed in the context of the Research Training Group «Theoretical Astrophysics» in Würzburg (Graduiertenkolleg, GK1147) shall be used. Particle acceleration is studied with hybrid MHD and particle-in-cell methods.

- The black hole mass and accretion rate will be determined from the emission models. Estimates of the black hole mass from emission models, a possible orbital modulation, and the Magorrian relation (relating the black hole mass with the stellar bulge mass of the host galaxy) will be compared.

- When flaring states will be discovered during the monitor program, MAGIC will issue a Target of Opportunity observation to obtain better time resolution (Letters of support?). Corresponding Target-of-Opportunity (ToO) proposals to HESS and Veritas are in preparation.

- Correlating the arrival times of neutrinos detected by the neutrino telescope IceCube with simultaneous measurements of DWARF will allow to test the hypothesis that flares in blazar jets are connected to hadronic emission processes and thus to neutrino emission from these sources. The investigation proposed here is complete for both, neutrino and gamma observations, and can therefore lead to conclusive results. Other attempts presently discussed, as MAGIC observations triggered by neutrino detection, are inappropriate because for the extraction of signal events from the background-dominated neutrino-sample the knowledge about correlation with the detected gamma emission is mandatory. Thus no conclusion about correlations can be drawn.
The Dortmund group is IceCube member and working since years on phenomenological calculations and data analysis of possible coincidences between VHE-gamma and neutrino-emission. (Letter of support see appendix)

- The diffusive fluxes of escaping UHE cosmic rays obtained from AUGER or flux limits of neutrinos from IceCube, respectively, will be used to constrain models of UHE cosmic ray origin and large-scale magnetic fields.

- Multi-frequency observations together with the Metsähovi Radio Observatory and the optical Tuorla Observatory are planned (Letters of support appendix). The measurements will be correlated with INTEGRAL and GLAST results, when available. X-ray monitoring using the SWIFT and Suzaku facilities will be proposed.

- The most ambitious scientific goal of this proposal is the search for signatures of binary black hole systems from orbital modulation of VHE gamma ray emission. In case of a confirmation of the present hints in the temporal behavior of Mrk501, gravitational wave templates could be computed with high accuracy to establish their discovery with LISA (PhD project at Würzburg funded by the German LISA consortium).

The technical setup: At the Observatorio de los Muchachos (ORM), at the MAGIC site, the mount of the former HEGRA telescope CT3 now owned by the MAGIC collaboration is still operational. Some parts need to be exchanged or repaired due to corrosion. Due to the steady progress in technology there is the possibility to slightly improve the performance of some components and thus raise the sensitivity with minor costs. One hut for electronics close to the telescope is available. Additional space is available in the MAGIC counting house.  The MAGIC Memorandum of Understanding allows for operating it as an auxiliary instrument, and basic support from the shift crew of MAGIC is guaranteed, although robotic operation is the primary goal. Robotic operation is necessary to reduce costs and man power demands. Furthermore, we seek to obtain know-how for the operation of future networks of Cherenkov telescopes (e.g. a monitoring array around the globe or CTA) or telescopes at inaccessible sites. From the experience with the construction and operation of MAGIC or HEGRA, respectively, the proposing groups consider the planned focused approach (small number of experienced scientists) as optimal for achieving the project goals. The available automatic analysis package developed by the Würzburg group for MAGIC is modular and flexible, and can thus be used with minor changes for the DWARF project. Therefore construction, commissioning and operation of a small scale Cherenkov telescope are best suitable for education and training of students by experienced scientists.

To complete the mount to a functional Cherenkov telescope within a period of one year, the following steps are necessary:

Camera:

- For long-term observations stability of the camera is a major criterion. To keep the systematic errors small good background estimation is mandatory. The only possibility for a synchronous determination of the background is the determination from the night-sky observed in the same field-of-view with the same instrument. To achieve this the observed position is moved out of the camera center which allows the estimation of the background from positions symmetric with respect to the camera center (so called wobble-mode). This observation mode increases the sensitivity by a factor of two because spending observation for dedicated background observations becomes obsolete, which also ensures a better time coverage of the observed sources. Having a camera large enough allowing more than one independent position for background estimation increases sensitivity further by better background statistics. This is the case if the source can be shifted 0.6deg-0.7deg out of the camera center. A camera completely containing shower images of events in the energy region of 1TeV-10TeV should have a diameter in the order of 5 deg. (hierzu haben wir glaube ich auch einen Plot) To decrease the dependence of the background measurement on the camera geometry, a camera layout as symmetric as possible will be chosen. Consequently a camera allowing for wobble-mode observations should be round and have a diameter of 4.5deg-5.0deg.
To achieve these requirements a 313 Pixel camera (see picture) will be build based on the experience with HEGRA and MAGIC. 19 mm diameter Photomultiplier Tubes (PM, EMI 4035) will be bought, similar to the HEGRA type (EMI 9083 KFLA). With a 20% improved quantum efficiency they ensure a granularity which is enough to guarantee good results even below the energy threshold (flux peak energy). Each individual pixel has to be equipped with a preamplifier, an active high-voltage supply and control. The total expense for a single pixel will be in the order of 600 €.

- If development of G-APDs (QE>=50%) will be fast enough, respectively the price low enough, and their long-term stability is proven well in time, their usage will be considered.

- For a transition time one of the old HEGRA cameras might be borrowed (with 271 pixels covering a field-of-view of nearly 5°). With a special coating (wavelength shifter) the PMT's quantum efficiency can be improved by passive means to ~8% (ref. Paneque).

Camera HT voltage system:

- The original HT system needs to be better adopted to the needs for robotic operation. Besides an increase in reliability it is necessary to improve the automatic current limitation in case of accidental illumination, for example by bright stars in the field-of-view or accidental human illumination. For longterm monitoring observations in the presence of modest moonshine is considered (Mkn 501 and 421 longterm monitoring also during moonshine was successfully pioneered by the HEGRA CT1 telescope).

Camera support:

- The camera chassis housing must be water tight. An automatic lid protecting the PMs at day-time will be installed. For further protection a Cytops (Polyfluorid plastic with high UV transmission and low surface reflectivity losses). By over-coating the window with an anti-reflex layer of magnesium-fluoride a gain in transmission of 5% is expected. Each PM will be equipped with a light-guide (Winston Cone) as developed by UC Davis and successfully in operation in the MAGIC camera. (3000 €). The current design will be improved by using a dielectric mirror foil with close to 100 % reflectivity. In combination with a new Cytops camera window this results in a gain in light collection efficiency by >=15% respectively threshold reduction compared to the old HEGRA CT3 telescope.

- For this setup the camera holding has to be redesigned. (1.500€?)

- An electric and optical shielding of the individual PMs is planned.

- The mechanical work is done at Universität Dortmund.

Data acquisition:

- The old data acquisition system, based on a home-made 100 Mhz system is very power hungry and not reliable enough for robotic operation. For the data acquisition system a hardware readout based on an analog ring buffer (Domino II/III), currently developed for the MAGIC II readout, will be used. A system based also on a Domino II ring sampler is already running since December at the -e- experiment at the PSI. This technology allows sampling the pulses with high frequencies and allows to readout several channels with a single Flash-ADC resulting in low-costs. The low power consumption will allow including the digitization near the signal source which makes an analog signal transfer obsolete. The advantage is less pick-up noise and less signal dispersion. By high sampling rates (0.5 GHz-1.2 GHz) additional information about the pulse shape can be obtained. This is increasing the over-all sensitivity further, because the short integration time allows for almost perfect suppression of noise due to night-sky background photons. The ring sampler has a depth of 1024 cells allowing to omit  delay cables needed for the matching of the trigger delay. This allows the operation of a higher level trigger logic, which can restrict the readout to only good candidate events, but increases trigger delay. At a sampling rate of 1 Ghz this methos allows for almost 800ns trigger delay without delay cables. The estimated trigger- (readout-) rate of the telescope is below 100 Hz (HEGRA: <10Hz) which allows to use a low-cost industrial solution for data transfer to the computer like USB2.0. (30.000-45.000€: 95-145€/Kanal). The entire ring sampler system fits easily into one VME crate (in principle 640 channels can fit into one crate). It is therefore forseen to install the data acquisition system within the telescope support structure. Compared to the past HEGRA configuration much shorter coax cables can be used resulting in quite reduced and less deformed transmission of the fast PMT signals. is forseen for inclusion into the camera chassis.

Figure XX shows some digitized PMT and g-apd signals with the Domino II system. The data shows Cherenkov light flashes recorded with the solar concentrator at the PSI. The observation conditions (very high light background) were considerably worse than what is expected at La Palma. Die fig aus dem poster der ersten Beobachtung von C pulsen (Vienna Inst Conf) nehmen.

- The past trigger electronics was prone to instabilities and had a low reliability. As for the HEGRA telescopes a simple multiplicity trigger is enough, but also a simple three-next-neighbors (closed package) could be programmed. (<30.000: <100/Kanal). A new concept can be copied in a simpler form from the new MAGIC design (MAGIC II) currently under discussion. Figure XXX shows the concept for one cell which is based on simple linear adder of the signals from seven pixels and a discriminator. In the past single large signals, for example from large afterpulses were causing many unwanted triggers. By adding clipping diodes in the summing network this can be prevented by a very simple method.

- To guarantee a homogenous trigger setup over the whole camera the individual pixel rates, dominated by night-sky noise, will be monitored and kept constant.

- Additional data reduction and preprocessing in the readout hardware or the readout computer is provided. Assuming conservatively storage of raw-data at a readout rate of 30Hz the storage space needed is less than 250GB/month or 3TB/year. This amount of data can easily be stored and processed by the Würzburg Datacenter (current online capacity >20TB, offline capacity >30TB, >16CPUs). To archive the data safely 25 tapes (LTO3 with 400GB each, ~1.000€) and a SATA disk-array (~4.000€) will be bought.

On-site computing:

- For on-site computing less than three standard PCs are needed (~8.000€). This includes readout and storage, preprocessing, and telescope control. For safety reasons a firewall is mandatory. For local storage and backup two RAID5 SATA disk arrays with less than one Terabyte capacity each will fulfill the requirement (~4.000€). The data will be transmitted as soon as possible after data taking via Internet to the Würzburg Datacenter.

- Monte Carlo production and storage will take place at Universität Dortmund

- For the absolute time necessary for an accurate source tracking a GPS clock will be bought.

Mount and Drive:

- The present mount is used. Only a smaller investment for safety, corrosion protection, cable ducts, etc. is needed (7.500).

- For movement motors, shaft encoders (14 bits absolute) and control electronics in the order of 10.000 € have to be bought. The drive system should allow for relatively fast repositioning for three reasons: 1) Fast movement might be mandatory for future ToO observations. 2) Wobble-mode observations will be done changing the wobble-position continuously (each 20min) for symmetry reasons. 3) To ensure good time coverage of more than one source visible at the same the observed source will be changed in constant time intervals (~20min). Therefore one servo motor (<=1kW) is intended for each axis. The rather simple drive electronics is mounted in a watertight box positioned inside the telescope undercarriage. The control computer just supplies the series of drive pulses, directional signal levels and controls of the range switches by industrial IO interface cards. The use of stepping motors in combination with low ‘play’ reduction gears allows one a very precise tracking even in the temporary absence of the shaft encoder information (as shown during the HEGRA operation). A microcontroller based motion control unit (SPS), similar to the one of the current MAGIC II drive system, will be used to operate the motors. For communication with the readout-system a standard Ethernet connection based on the TCP/IP- and UDP-protocol is applied. This allows for a simple control program which can be run within the data acquisition and omits non-standard I/O hardware thus improving the reliability of the system.

Security:

- An uninterruptible power-supply unit (UPS) with 5-10kW will be installed to protect the equipment against power cuts and ensure a safe telescope position at the time of sun-rise. (<2.000€)

Mirrors:

- The existing mirrors are replaced by new plastic mirrors which are currently developed by the group of Wolfgang Dröge. The cheap and light-weight material has been formerly used for Winston cones flown in balloon experiments. The mirrors are copied from a master, coated with a reflecting and a protective material. Previous tests have given promising results. By a change of the mirror geometry the mirror area can be increased from 8.5 m2 to 13m2 (see pictures); this includes an increase of ~10% per mirror by using a hexagonal layout. A further increase of the mirror area would require a reconstruction of parts of the mount and will therefore be considered only in later phase of the experiment. 
If the current development cannot be finished in time a re-machining of the old glass mirrors (8.5m²) is possible with high purity aluminum and quartz coating. (Both cases: 30 mirrors, 10k, offer by L-Tec ~<500 € / mirror *30 mirrors = 15.000 € without transfer)

- To keep track of the alignment, reflectivity and optical quality of the individual mirrors, and the point-spread function of the total mirror, during long-term observations the application of an automatic mirror adjustment system, as developed by ETH Zurich and successfully operated on the MAGIC telescope, is intended. The system will be provided by ETH Zürich. (1.000 €/pannel)

- For a 3.5 m diameter mirror the delay between an isochronous parabolic mirror and a spherical mirror at the edge is in the order of 1ns (see figure/appendix). For a sampling rate in the order of 1GHz a mirror mounting with a parabolic shape in needed. Since their small size the individual mirrors can still have a spherical shape.

Telescope calibration:

- Tracking: To correct for axis misalignments and possible deformations of the structure (e.g. bending of camera holding masts) a pointing correction algorithm as used in the MAGIC tracking system will be applied. It is calibrated by measurement of the reflection of bright guide stars on the camera surface and ensures a pointing accuracy well below the pixel diameter. Therefore a high sensitive low-cost video camera, as already in operation for MAGIC I and II, (300 € camera, 300 € optics, 300 € housing) will be installed.

- PM Gain: For the calibration of the PM gain a calibration system as used for the MAGIC telescope is build. (2.000 €)

Extra safety installations for robotic operation  and safety ?????

If no precise numbers but ranges are given in the text further more detailed Monte Carlo studies are still ongoing.

Summarizing, the expenses for the telescope are dominated by the camera and DAQ. The financial volume for the complete hardware inclusive transport amounts roughly 400.000 €.

Future extensions: The known duty cycle of 10% (~1.000h/year) for a Cherenkov telescope operated at La Palma limits the time-coverage of the observations. Therefore we propose a worldwide network of (<10) small scale Cherenkov telescopes to be build in the future allowing 24h monitoring of the bright AGNs. Such a system is so far completely unique in this energy range.

In a first stage of the project mounts of other former HEGRA telescopes could be used operated at locations in Croatia, the United States and Mexico. For an increased sensitivity and improved energy threshold the use of a low-cost mount build by the company MERO for solar power generation is proposed. The mount is based on the experiences with the MAGIC telescope, also built by MERO, and has a diameter in the order of eight meters. Including support (concrete foundation, railways, etc) the costs are below 100.000€.

Kommentare von Eckart in rot.

Kommentare von Eckart zu Punkten, die es im aktuellen LoI nicht mehr gibt in violet.

Punkte, die es in der von Eckart kommentierten Version noch nicht gab in himmelblau.

Im aktuellen LoI anzuhängenden appendices in grün.

Costs:

- Camera: 313Pixel*650€/Pixel ~ 200.000€ (204k€)

- Pixel: 650€/Pixel

- 300-350€€ Photomultiplier (EMI 4051)

- 50€ Preamplifier

- 200-250€ HV control and support (EMI)

- Winston Cones: 3000€ (?)

- Camera holding and chassis: 3000€(?)

- Data acquisition: 313channel*245€/channel ~ 77.000€

- 145 (95) €/channel Readout

- 100€/channel Trigger

- Calibration System: 9.000€

- 2000€ Absolute light calibration?

- IPR control?

- Weather station 500€

- 1500€ GPS clock

- 5.000€ CD Cameras + readout

- Mirrors: 15.000€

- On-site computing: 12.000€

- 3xPC: 8000€

- SATA RAID 3TB: 4000€

- Computing: 5.000€

- 25 LTO3 Tapes: 1000€

- 3TB SATA Disk space: 4000€(?)

- AMC: 1000€/pannel

- UPS: 2000€

- Minor expenses: 60.000€

- 5.000€ transport and storage container

- operation costs: 5000€/3years

- 7.500€ Robotics

- 35.000€ Travel and construction

- 10.000€ Consumables

- Dismantling (0, will be covered by proposing institutes)

- 15.000 Transport

Total: ~ 400.500€